# Annotated Code Document

Jacob Lamoureux

Project 3

March 31, 2026

| # Code | # Annotation |
| --- | --- |
| import pandas as pd | Imports the pandas library for loading and working with tabular data (CSV files). Aliased as 'pd' for brevity. |
| import numpy as np | Imports the numpy library for numerical array operations used in the IDM simulation. Aliased as 'np'. |
| import matplotlib.pyplot as plt | Imports the pyplot module from matplotlib, the core library used to build and display all plots. Aliased as 'plt'. |
| import seaborn as sns | Imports the seaborn library, which provides higher-level statistical plotting functions built on top of matplotlib. Aliased as 'sns'. |
| sns.set_style('whitegrid') | Applies a clean white background with grid lines to all subsequent plots, improving readability. |
| sns.set_palette('deep') | Sets the default color palette for all seaborn plots to the 'deep' palette. |
| ngsim = pd.read_csv('NGSIM.csv') | Reads the NGSIM CSV file into a pandas DataFrame called 'ngsim'. Contains time-series vehicle trajectory data including position, speed, and acceleration. |
| traveldata = pd.read_csv('NHTS.csv') | Reads the NHTS CSV file into a pandas DataFrame called 'traveldata'. Contains household-level transportation survey data including vehicle characteristics and demographics. |
| fuelcounts = traveldata['fuel_type'].value_counts().dropna() | Selects the 'fuel_type' column, counts how many vehicles fall into each category, and removes any rows with missing values. Stores the result as a Series called 'fuelcounts'. |
| plt.figure(figsize=(6,4)) | Creates a new figure canvas with a width of 6 inches and height of 4 inches. |
| fuelcounts.plot(kind='bar', edgecolor='black', color=sns.color_palette('viridis', len(fuelcounts))) | Plots 'fuelcounts' as a bar chart. 'edgecolor' adds black outlines to each bar. The viridis color palette is applied across bars using seaborn, with the number of colors matching the number of fuel type categories. |
| plt.xlabel('Fuel Type', fontsize=12) | Labels the x-axis. |
| plt.ylabel('Count', fontsize=12) | Labels the y-axis. |
| plt.title('Vehicle Count by Fuel Type', fontsize=16) | Adds a descriptive title to the plot. |
| plt.xticks(fontsize=9) | Sets the font size of x-axis tick labels. |
| plt.yticks(fontsize=9) | Sets the font size of y-axis tick labels. |
| plt.xticks(rotation=65) | Rotates x-axis labels 65 degrees to prevent category names from overlapping. |
| plt.show() | Renders and displays the finished plot. |
| plt.figure(figsize=(6,4)) | Creates a new figure canvas, same as before. |
| plt.hist(traveldata['vehicle_age'], bins=40, color='mediumaquamarine', edgecolor='lightgoldenrodyellow') | Plots a histogram of the 'vehicle_age' column split into 40 bins. The fill color and edge color are both set manually to demonstrate matplotlib color customization. |
| plt.xlabel('Vehicle Age', fontsize=12) | Labels the x-axis. |
| plt.ylabel('Count', fontsize=12) | Labels the y-axis. |
| plt.title('Vehicle Age Distribution', fontsize=16) | Adds a descriptive title. |
| plt.show() | Renders and displays the plot. |
| plt.figure(figsize=(16,10)) | Creates a wide figure to give enough horizontal space for all vehicle make labels along the x-axis. |
| boxplot = traveldata.boxplot(column='vehicle_age', by='make') | Uses pandas' built-in boxplot method to plot the distribution of 'vehicle_age' grouped by each unique vehicle 'make'. Each box shows the median, interquartile range, whiskers, and outliers for that make. |
| plt.xticks(rotation=90) | Rotates x-axis make labels 90 degrees so they are readable without overlapping. |
| plt.xlabel('Make', fontsize=10) | Labels the x-axis. |
| plt.ylabel('Vehicle Age (years)', fontsize=10) | Labels the y-axis. |
| plt.title('Boxplot of Vehicle Age by Make', fontsize=10) | Adds a descriptive title. |
| plt.suptitle('') | Removes the automatic subtitle that pandas adds above boxplots by default, which would otherwise clutter the figure. |
| plt.show() | Renders and displays the plot. |
| trajectorynumber = 10 | Sets the trajectory number to analyze. Each trajectory number represents a unique leader-follower vehicle pair in the NGSIM dataset. |
| data_subset = ngsim[ngsim['trajectory_number'] == trajectorynumber] | Filters the full NGSIM DataFrame to only rows where 'trajectory_number' equals 10, isolating one vehicle pair. |
| gap = data_subset['leader_position(m)'].values - data_subset['follower_position(m)'].values | Derives the gap distance at each time step by subtracting the follower's position from the leader's position. This variable is not in the original dataset and is computed here. |
| time = data_subset['Time'].values | Extracts the time array from the subset as a numpy array, used as the x-axis for all four subplots. |
| fig, axes = plt.subplots(2, 2, figsize=(10,10)) | Creates a figure containing a 2x2 grid of subplot panels. 'fig' refers to the overall figure and 'axes' is a 2D array of the four individual subplot panels. |
| axes[0,0].plot(time, data_subset['leader_position(m)'].values, linewidth=1.5, label='Leader', color='slateblue') | Plots the leader vehicle's position over time in the top-left panel as a blue line. |
| axes[0,0].plot(time, data_subset['follower_position(m)'].values, linewidth=1.5, label='Follower', color='tomato') | Plots the follower vehicle's position over time on the same top-left panel as a red line. |
| axes[0,0].set_ylabel / set_xlabel / set_title / legend | Adds axis labels, a title, and a legend to the top-left subplot panel. |
| axes[1,0].plot(...leader_speed...) / axes[1,0].plot(...follower_speed...) | Same structure as above, but plots leader and follower speed over time in the bottom-left panel. |
| axes[1,0].set_ylabel / set_xlabel / set_title / legend | Same as before, labels the bottom-left speed subplot. |
| axes[0,1].plot(...leader_acc..., alpha=0.5) / axes[0,1].plot(...follower_acc..., alpha=0.75) | Plots leader and follower acceleration over time in the top-right panel. The 'alpha' argument makes the lines semi-transparent so overlapping data is still visible. |
| axes[0,1].set_ylabel / set_xlabel / set_title / legend | Same as before, labels the top-right acceleration subplot. |
| axes[1,1].plot(time, gap, ...) | Plots the derived gap distance over time in the bottom-right panel. |
| axes[1,1].set_ylabel / set_xlabel / set_title / legend | Same as before, labels the bottom-right gap subplot. |
| fig.suptitle('Trajectory Analysis', fontsize=20) | Adds a single overall title centered above all four subplots. |
| plt.tight_layout() | Automatically adjusts spacing between subplots so titles and labels do not overlap. |
| plt.show() | Renders and displays the complete dashboard figure. |
| def idm_acceleration(v, s, delta_v, v0, s0, T, a, b, delta): | Defines a Python function named 'idm_acceleration'. It accepts the follower's current speed (v), gap distance (s), relative speed (delta_v), and all IDM model parameters as inputs. |
| s_star = s0 + v * T + (v * delta_v) / (2 * np.sqrt(a * b)) | Computes the desired minimum gap (s*) using the IDM formula. This represents the gap the driver wants to maintain given their current speed and approach rate. |
| s_star = max(s_star, s0) | Ensures s_star never falls below the minimum spacing s0, preventing physically impossible or mathematically unstable values. |
| acceleration = a * (1 - (v/v0)**delta - (s_star/s)**2) | Applies the core IDM equation to compute acceleration. The result is positive when the vehicle wants to speed up and negative when it wants to brake. |
| return acceleration | Returns the computed acceleration value to wherever the function was called. |
| datasubset = ngsim[ngsim['trajectory_number'] == trajectorynumber].reset_index(drop=True) | Filters NGSIM to the same trajectory number chosen in Cell 5, then resets the row index to start at 0 so array indexing works correctly in the simulation loop. |
| timedata = datasubset['Time'].values | Extracts the time array as a numpy array for use as the simulation x-axis. |
| leaderposition = datasubset['leader_position(m)'].values | Extracts the leader vehicle's position at each time step. Used each iteration to compute the gap between leader and simulated follower. |
| leaderspeed = datasubset['leader_speed(m/s)'].values | Extracts the leader vehicle's speed at each time step. Used to compute the relative speed (delta_v) each iteration. |
| dt = 0.1 | Sets the simulation time step to 0.1 seconds, matching the sampling rate of the NGSIM data. |
| v0 = 30 | Desired free-flow velocity of the simulated driver (m/s), approximately 67 mph. |
| s0 = 2 | Minimum bumper-to-bumper spacing the driver maintains even when stopped (m). |
| T = 1.5 | Desired time headway, the time gap the driver wants to maintain behind the leader (s). |
| a = 1.0 | Maximum comfortable acceleration of the simulated vehicle (m/s^2). |
| b = 1.5 | Maximum comfortable braking deceleration (m/s^2). |
| delta = 4 | Acceleration exponent that controls how sharply the driver reduces acceleration as speed approaches the desired velocity. A value of 4 is standard in IDM literature. |
| nsteps = len(timedata) | Sets the number of simulation steps equal to the number of time steps in the NGSIM trajectory data. |
| simposition = np.zeros(nsteps) | Creates an array of zeros with length 'nsteps' to store the simulated follower's position at each time step. |
| simspeed = np.zeros(nsteps) | Same structure, stores simulated follower speed at each time step. |
| simacc = np.zeros(nsteps) | Same structure, stores simulated follower acceleration at each time step. |
| simposition[0] = datasubset['follower_position(m)'].values[0] | Sets the first element of the position array to the real follower's starting position from NGSIM, so the simulation begins from the same point as the real vehicle. |
| simspeed[0] = datasubset['follower_speed(m/s)'].values[0] | Same as above, initializes the simulation at the real follower's starting speed. |
| for i in range(nsteps - 1): | Loops through each time step from 0 to nsteps-2. Stops one step before the end because each iteration writes to index i+1. |
| gap = max(leaderposition[i] - simposition[i], 0.1) | Computes the gap between the real leader and simulated follower at time step i. The floor of 0.1m prevents division by zero in the IDM equation if the vehicles were ever at the same position. |
| delta_v = simspeed[i] - leaderspeed[i] | Computes the relative speed of the follower with respect to the leader. A positive value means the follower is approaching the leader. |
| simacc[i] = idm_acceleration(simspeed[i], gap, delta_v, v0, s0, T, a, b, delta) | Calls the IDM function defined in Cell 6 with the current state values and parameters, storing the resulting acceleration. |
| simspeed[i+1] = max(simspeed[i] + simacc[i] * dt, 0) | Updates the follower's speed for the next time step using Euler integration (new = old + rate * dt). The max(..., 0) prevents the speed from going negative. |
| simposition[i+1] = simposition[i] + simspeed[i] * dt | Updates the follower's position for the next time step using Euler integration. |
| print('Simulation complete!') | Prints a confirmation message to verify the loop ran without errors. |
| plt.figure(figsize=(12,6)) | Creates a wide figure to clearly show the trajectory comparison over time. |
| plt.plot(timedata, leaderposition, label='Leader (NGSIM)', color='deeppink') | Plots the real leader vehicle's position from NGSIM data. |
| plt.plot(timedata, datasubset['follower_position(m)'].values, label='Follower (NGSIM)', color='goldenrod') | Plots the real follower vehicle's position from NGSIM data, for comparison against the simulation. |
| plt.plot(timedata, simposition, label='Simulated Follower', color='mediumaquamarine') | Plots the IDM-simulated follower position. Comparing this line to the NGSIM follower line shows how accurately the IDM model replicates real driving behavior. |
| plt.xlabel('Time (seconds)') | Labels the x-axis. |
| plt.ylabel('Position (m)') | Labels the y-axis. |
| plt.title('IDM Simulation vs NGSIM Trajectory') | Adds a descriptive title. |
| plt.legend() | Adds a legend to identify each of the three plotted lines. |
| plt.grid(True, alpha=0.3) | Adds a light grid to the background of the plot to aid in reading values. Alpha of 0.3 keeps it subtle. |
| plt.show() | Renders and displays the finished plot. |