# Scope of Work

Jacob Lamoureux

Project 3

March 31, 2026

Table of Contents

Project Summary	3

Tasks Completed	3

NHTS Variables	3

NGSIM Variables	4

Deliverables	4

# Project Summary

The purpose of this project is to organize and visualize transportation data from two federally collected datasets, the National Household Travel Survey (NHTS) and the Next Generation Simulation (NGSIM) dataset, on behalf of a transportation planning firm. The work includes loading and inspecting both datasets, producing a series of meaningful visualizations, and running a vehicle following simulation using the Intelligent Driver Model (IDM). All work was completed individually using Python.

# Tasks Completed

To complete this project, a GitHub repository was first set up to store all project files including the code, data, and documentation. Both the NHTS and NGSIM datasets were then loaded and inspected using pandas, checking for missing values and understanding the structure of each dataset before any plotting was done. From the NHTS dataset, a bar chart, histogram, and boxplot were produced to summarize key vehicle characteristics. Time-series plots were then built from the NGSIM dataset showing position, speed, acceleration, and gap distance for a selected leader-follower vehicle pair. The Intelligent Driver Model was implemented in Python and a simulation was run using real NGSIM leader trajectory data, with the simulated follower position compared against the real recorded follower position. Finally, an annotated code document was written to explain each line of the final code, and time spent on each task was recorded.

# NHTS Variables

The NHTS dataset contains household-level survey data collected by the Federal Highway Administration. Three variables were selected for visualization because they provide the most relevant insight into vehicle fleet composition and ownership trends in the US.

The first variable is fuel_type, which describes what type of fuel each surveyed vehicle uses. Categories include gas, diesel, electric, hybrid, plug-in hybrid, and biodiesel. This variable was selected for the bar chart because fuel type is one of the most policy-relevant characteristics of the current vehicle fleet, particularly as the transportation industry shifts toward electrification.

The second variable is vehicle_age, which records the age of each surveyed vehicle in years. This variable was selected for both the histogram and the boxplot because it is a continuous variable that reveals the overall age of the US vehicle fleet. Older fleets can indicate lower household income, reduced access to new vehicles, or slower adoption of newer fuel technologies.

The third variable is make, which identifies the manufacturer of each surveyed vehicle. This variable was used as the grouping category in the boxplot to show how vehicle age varies across different manufacturers, which can reveal differences in brand loyalty and vehicle longevity.

# NGSIM Variables

The NGSIM dataset contains time-series records of paired leader and follower vehicles collected on US highways. Several variables from this dataset were used for the time-series visualizations and the IDM simulation.

The Time variable records the timestamp in seconds for each observation and was used as the x-axis for all time-series plots. The position variables, leader_position(m) and follower_position(m), record the highway position of each vehicle in meters at each time step and were used to plot vehicle trajectories over time. From these two position columns, a gap distance variable was derived by subtracting the follower position from the leader position at each time step. This derived variable represents the bumper-to-bumper spacing between the two vehicles and is one of the most important variables in car-following analysis.

The speed variables, leader_speed(m/s) and follower_speed(m/s), record each vehicle's speed in meters per second and were plotted to show how closely the follower mirrors the behavior of the leader over time. Similarly, the acceleration variables, leader_acc(m/s^2) and follower_acc(m/s^2), record acceleration in meters per second squared and reveal braking and accelerating events throughout the observation period.

Finally, trajectory_number is an identifier for each unique leader-follower vehicle pair in the dataset. The dataset contains 16 trajectories, and trajectory 10 was selected for the visualizations and simulation.

# Deliverables

The deliverables for this project include a GitHub repository containing all code, data, and documentation, a final Python notebook with all visualizations and the IDM simulation, an annotated code document, a written methods document, this scope of work, and a Gantt chart with timesheet.